Design and Development of a Microcontroller-Based Robotic Arm
An IT Project Proposal
Presented to the Faculty of the
College of Information and Computing Sciences
Zamboanga Peninsula Polytechnic State University

In Partial Fulfillment of the Requirements
for the degree in Bachelor of Science in Information Technology

By
Alim, Al-khazri S.
Pagotaisidro, Marco Jean
Ramillano, Incent

Glaiza Mustapha
Adviser

2025

Chapter I
INTRODUCTION

The integration of computing systems with the physical world through automation and robotics represents a significant frontier in information technology. Microcontroller-based robotic arms are a fundamental application of this integration, demonstrating principles of mechatronics, programming, and real-time control. For educational and light industrial purposes, developing a low-cost, programmable robotic arm provides a practical platform for understanding these core concepts. This project focuses on the design and construction of a functional robotic arm prototype using an ESP32 microcontroller and servo motors.

Currently, many educational institutions and small-scale workshops face challenges in accessing affordable, hands-on training tools for robotics. Commercially available robotic arms are often expensive and proprietary, limiting customization and deep understanding. The development of this project addresses these concerns by presenting a structured, component-based approach to building a robotic arm, emphasizing the interconnection of hardware and software.

This research focuses on designing a 3-Degree-of-Freedom (3-DOF) robotic arm model. The proposed design aims to demonstrate how an ESP32 microcontroller can precisely control multiple servo motors to manipulate the arm's movement, enabling tasks such as picking and placing lightweight objects. The system will be programmable, allowing for a sequence of movements to be stored and executed.

This study is conducted to conceptualize and implement a functional robotic arm prototype that demonstrates the practical application of IT in mechatronics. The project intends to illustrate how proper system design, component selection, and programming can result in a reliable automated system. It also serves as a formal requirement for the course, demonstrating the students' understanding of embedded systems and hardware integration as applied to a tangible real-world scenario.

Chapter II
RELATED LITERATURE

The field of robotics has evolved significantly, with microcontroller-based systems becoming increasingly accessible for prototyping and education. Literature shows that the success of such systems hinges on the effective integration of mechanical design, electronic control, and software logic.

According to Bolton (2015), the design of a mechatronic system must follow a structured approach, considering kinematics, actuator selection, and control system design. The choice of actuators, such as servo motors, is critical for precise angular positioning, which is fundamental for robotic arm articulation. For low-torque applications, standard hobby servos provide a cost-effective and readily controllable solution (Craig, 2005).

The role of the microcontroller as the "brain" of the system is emphasized by Wilmshurst (2016). Modern microcontrollers like the ESP32 offer advantages such as a dual-core processor, integrated Wi-Fi/Bluetooth, and multiple Pulse Width Modulation (PWM) channels, making them ideal for controlling multiple servos and enabling potential IoT connectivity (ESP32 Datasheet, 2022). PWM is the standard control signal for hobby servos, allowing for precise control of the shaft position based on the pulse width.

Studies on robotic arm kinematics highlight the importance of the Degrees of Freedom (DOF). A 3-DOF arm, while limited in complexity, is sufficient to demonstrate fundamental movements in a single plane (pick-and-place), making it an excellent educational model (Siciliano & Khatib, 2016). The mechanical structure must be rigid enough to handle the loads without excessive flexure, which influences the choice of materials like acrylic or 3D-printed plastics.

Furthermore, the software architecture is crucial for smooth operation. Programming environments like the Arduino IDE (which supports the ESP32) simplify the development process. The code must manage the timing and sequencing of servo movements to create coordinated motion, often implemented through state machines or predefined movement arrays (Jones, 2020).

Based on the reviewed literature, the design for the robotic arm must emphasize a stable mechanical structure, precise servo control, and logical, well-structured software. These principles serve as the foundation for the succeeding chapters of this study.

The selection of components is a critical element in designing an efficient and cost-effective robotic arm. According to Wilmshurst (2016), the microcontroller must have sufficient I/O pins and PWM capabilities to drive all actuators without external multiplexing. The ESP32, with its numerous GPIO pins and PWM support on all output-capable pins, meets this requirement effectively.

Servo motors are the primary actuators for small robotic arms. As described by Craig (2005), servos integrate a motor, a gearbox, and a feedback control system that allows them to move to and hold a specific angular position. For a 3-DOF arm, at least three servos are required: one for the base rotation, one for the shoulder, and one for the elbow. A fourth servo can be added for a functional gripper.

Power management is another vital consideration. Servo motors can draw significant current, especially under load. Therefore, a dedicated external power supply for the servos, separate from the microcontroller's power, is recommended to prevent brownouts and ensure stable operation (Bolton, 2015).

Overall, the literature supports the use of an ESP32 microcontroller and standard servo motors for constructing a low-cost, programmable, and effective robotic arm for educational and prototyping purposes.

Chapter III
METHODOLOGIES/STRATEGIES

The development of the proposed robotic arm followed several structured methodologies commonly used in embedded systems and mechatronics projects:

- Needs Analysis and Requirement Specification
A functional analysis was conducted to define the arm's capabilities:

  - Must have at least 3 Degrees of Freedom (Base, Shoulder, Elbow).

  - Must be able to grip and lift a lightweight object (e.g., an empty plastic bottle).

  - Must be controllable via programmed sequences.

  - Components must be low-cost and readily available.

- System Design and Component Selection

  - Microcontroller: ESP32 development board was selected for its sufficient I/O pins, PWM capabilities, and built-in wireless features for future expansion.

  - Actuators: Standard SG90 or MG90S hobby servo motors were chosen for their balance of torque, cost, and size.

  - Structure: The arm's body and links were designed for 3D printing or laser cutting from acrylic, ensuring lightness and rigidity.

  - Power: A 5V/3A DC power supply was selected to power the servos, while the ESP32 is powered via USB or the 5V rail.

  - Connections: Jumper wires and a breadboard were used for initial prototyping.

- Mechanical Assembly
The arm was assembled in a hierarchical manner:

  - The base servo was fixed to the base structure.

  - The shoulder servo was attached to a link connected to the base.

  - The elbow servo was attached to the subsequent link.

  - A gripper mechanism was attached to the final link, actuated by a fourth servo.

- Circuit Wiring and Connections
The electronic components were connected as follows:

  - The control signal pin (PWM) of each servo was connected to a dedicated digital pin on the ESP32 (e.g., GPIO 13, 12, 14, 27).

  - The VCC and GND of all servos were connected to the common 5V power supply, with capacitors used for noise filtering.

  - The ESP32's GND was connected to the common ground of the power supply.

- Software Development and Programming
The ESP32 was programmed using the Arduino IDE:

  - The Servo.h library was included to simplify servo control.

  - Servo objects were created for each motor.

  - Functions were written to map angular positions to specific PWM values.

  - A sequence of movements (a "pick and place" routine) was coded by defining a series of target angles for each servo with controlled delays between movements.

- Testing and Calibration
The system underwent iterative testing:

  - Each servo was tested individually to verify its range of motion.

  - The kinematic sequence was tested step-by-step.

  - Servo angles were calibrated to ensure the arm moves to precise, intended positions.

Chapter IV
Results and Discussion

The design and development process resulted in a functional and programmable 3-DOF robotic arm prototype. The implementation successfully demonstrates the integration of hardware and software to create an automated mechatronic system.

- Mechanical Assembly and Structure
The 3D-printed/acrylic structure provided a lightweight yet sufficiently rigid frame for the robotic arm. All servos were securely mounted, allowing for smooth rotation at each joint (base, shoulder, and elbow). The gripper mechanism effectively opened and closed, capable of gripping lightweight objects. The assembly proved stable during operation, with minimal unwanted flexing.

- Electronic Control and Connectivity
The wiring scheme using the ESP32 microcontroller was successfully implemented:

  - The ESP32 provided stable PWM signals to all four servos simultaneously.

  - The use of an external 5V power supply for the servos prevented voltage drops and ensured the ESP32 operated without resets or interference.

  - All connections remained secure throughout the testing phase.

- Software Functionality and Movement Control
The programmed code executed flawlessly:

  - The Servo.h library allowed for precise control over each servo's position.

  - The arm successfully performed a pre-programmed "pick and place" routine. This involved moving from a "home" position, lowering the gripper, closing it to grasp an object, lifting the object, moving it to a new location, and releasing it.

  - The movements were sequential and repeatable, demonstrating the reliability of the control program.

- System Performance
The robotic arm met its primary operational goals:

  - It achieved 3 Degrees of Freedom plus a functional gripper.

  - It could reliably pick up and place a lightweight object (a small empty box).

  - The entire system responded correctly to the programmed sequence without manual intervention.

- Challenges and Solutions

  - Challenge: Initial jittery movement in some servos.

  - Solution: This was mitigated by adding decoupling capacitors across the servo power lines and ensuring a robust, stable power supply.

  - Challenge: Calibrating the exact angles for precise positioning.

  - Solution: An iterative process was used, testing and adjusting the angle values in the code until the arm moved to the desired locations.

- Demonstration and Educational Value
The robotic arm prototype successfully demonstrated:

  - The practical application of microcontroller programming.

  - The principles of PWM for motor control.

  - The fundamentals of robotic kinematics and mechatronic system design.

  - A tangible example of how IT skills can be applied to control physical hardware.

- Conclusion
The implemented robotic arm design effectively met the goal of creating a low-cost, functional, and programmable automation tool. The project showcased a successful integration of mechanical design, electronic interfacing, and software logic. The results highlight how a structured approach to embedded systems projects can yield a functional prototype capable of performing a defined task, providing a solid foundation for understanding more complex robotic systems.